Lab 3 Part 2 - Configuring AAA Authentication on Routers

# Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A | S1 F0/1 |
|  | S0/0/0 (DCE) | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 | N/A | S2 F0/2 |
|  | S0/0/0 | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
|  | S0/0/1 (DCE) | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| R3 | G0/1 | 192.168.3.1 | 255.255.255.0 | N/A | S3 F0/5 |
|  | S0/0/1 | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| TACACS+ Server | NIC | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 | S2 F0/6 |
| RADIUS Server | NIC | 192.168.3.2 | 255.255.255.0 | 192.168.3.1 | S3 F0/1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S1 F0/2 |
| PC-B | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 | S2 F0/1 |
| PC-C | NIC | 192.168.3.3 | 255.255.255.0 | 192.168.3.1 | S3 F0/18 |

Objectives

Configure a local user account on R1 and configure authenticate on the console and vty lines using local AAA.

Verify local AAA authentication from the R1 console and the PC-A client.

Configure server-based AAA authentication using TACACS+.

Verify server-based AAA authentication from the PC-B client.

Configure server-based AAA authentication using RADIUS.

Verify server-based AAA authentication from the PC-C client.

Background / Scenario

The network topology shows routers R1, R2 and R3. Currently, all administrative security is based on knowledge of the enable secret password. Your task is to configure and test local and server-based AAA solutions.

You will create a local user account and configure local AAA on router R1 to test the console and vty logins.

User account: Admin1 and password admin1pa55

You will then configure router R2 to support server-based authentication using the TACACS+ protocol. The TACACS+ server has been pre-configured with the following:

Client: R2 using the keyword tacacspa55

User account: Admin2 and password admin2pa55

Finally, you will configure router R3 to support server-based authentication using the RADIUS protocol. The RADIUS server has been pre-configured with the following:

Client: R3 using the keyword radiuspa55

User account: Admin3 and password admin3pa55

The routers have also been pre-configured with the following:

Enable secret password: ciscoenpa55

OSPF routing protocol with MD5 authentication using password: MD5pa55

Note: The console and vty lines have not been pre-configured.

Note: IOS version 15.3 supports SCRYPT as a secure encryption hashing algorithm; however, the IOS version that is currently supported in Packet Tracer uses MD5. Always use the most secure option available on your equipment.

Configure Local AAA Authentication for Console Access on R1

Test connectivity.

Ping from PC-A to PC-B.

Ping from PC-A to PC-C.

Ping from PC-B to PC-C.

QUESTION 1 Which of the pings, if any, were successful?

- Before proceeding, go to the “Lab 3 Part 2 - Configuring AAA Authentication on Routers - QUESTIONS” quiz on the Brightspace page and enter your answer for question 1. Leave the quiz open while you complete the rest of the lab sheet.

Configure a local username on R1.

Configure a username of Admin1 with a secret password of admin1pa55.

Enable AAA on R1.

From global configuration mode on R1, enter the command aaa authentication login TEST enable.

R1(config)# aaa authentication login TEST enable

QUESTION 2 Was the command successful?

- Before proceeding, return to the quiz on the Brightspace page and enter your answer for question 2. Leave the quiz open while you complete the rest of the lab sheet.

Enable AAA on R1.

Configure local AAA authentication for console access on R1.

Configure AAA authentication for logins to use the local database by default. Ensure the login entries are case sensitive.

Configure the line console to use the defined AAA authentication method.

Configure the console login on R1 to use the default method list for authentication.

Verify the AAA authentication method.

Verify the user EXEC login using the local database. Try logging in using the wrong case in the username or password and note the results.

Configure Local AAA Authentication for vty Lines on R1

Configure domain name and crypto key for use with SSH.

Use netsecurity.com as the domain name on R1.

Create an RSA crypto key using 1024 bits.

Configure a named list AAA authentication method for the vty lines on R1.

Configure a named list called LOGIN-SSH to authenticate logins using local AAA. The logins do not need to be case sensitive.

Configure the vty lines to use the defined AAA authentication method.

Configure the vty lines to use the named AAA method and only allow SSH for remote access.

Verify the AAA authentication method.

Verify the SSH configuration SSH to R1 from the command prompt of PC-A.

QUESTION 3 Was PC-A able to successfully SSH to R1?

- Before proceeding, return to the quiz on the Brightspace page and enter your answer for question 3. Leave the quiz open while you complete the rest of the lab sheet.

Configure Server-Based AAA Authentication Using TACACS+ on R2

Configure a backup local database entry called Admin.

For backup purposes, configure a local username of Admin2 and a secret password of admin2pa55.

Verify the TACACS+ Server configuration.

Click the TACACS+ Server.  On the Services tab, click AAA. Notice that there is a Network configuration entry for R2 and a User Setup entry for Admin2.

Configure the TACACS+ server specifics on R2.

Configure the AAA TACACS server IP address and secret key on R2.

Note: The commands tacacs-server host and tacacs-server key are deprecated. Currently, Packet Tracer does not support the new command tacacs server.

R2(config)# tacacs-server host 192.168.2.2

R2(config)# tacacs-server key tacacspa55

Configure AAA login authentication for console access on R2.

Enable AAA on R2 and configure all logins to authenticate using the AAA TACACS+ server. If it is not available, then use the local database (not case sensitive).

Configure the line console to use the defined AAA authentication method.

Configure AAA authentication for console login to use the default AAA authentication method.

Verify the AAA authentication method.

Verify the user EXEC login using the AAA TACACS+ server.

Configure Server-Based AAA Authentication Using RADIUS on R3

Configure a backup local database entry called Admin.

For backup purposes, configure a local username of Admin3 and a secret password of admin3pa55.

Verify the RADIUS Server configuration.

Click the RADIUS Server. On the Services tab, click AAA. Notice that there is a Network configuration entry for R3 and a User Setup entry for Admin3.

Configure the RADIUS server specifics on R3.

Enter radius server configuration mode on R3 using RAD-SERVER as the server name. Configure the AAA RADIUS server IP address of 192.168.3.2 and secret key of radiuspa55. (Do not change the authentication/accounting ports used as the server has already been pre-configured to use 1645.)

Note: Packet Tracer does support the new command radius server.

Configure AAA login authentication for console access on R3.

Enable AAA on R3 and configure all logins to authenticate using the AAA RADIUS server. If it is not available, then use the local database (case sensitive).

Configure the line console to use the defined AAA authentication method.

Configure AAA authentication for console login to use the default AAA authentication method.

Verify the AAA authentication method.

Verify the user EXEC login using the AAA RADIUS server.

If you have correctly configured all parts of the lab your activity score should now be showing as 100%. If so, click on “check results” in the activity window. Return to the Brightspace quiz one last time and enter the code into the appropriate question box (Q4) of the quiz.

You have completed the lab – please submit the Brightspace quiz.