Lab 4 - Configuring OSPF Authentication, NTP, Syslog and SSH

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| RT1 | G0/0/1 | 172.16.1.1 | 255.255.255.0 | N/A |
|  | S0/1/0 | 10.1.1.1 | 255.255.255.252 | N/A |
| RT2 | S0/1/0 (DCE) | 10.1.1.2 | 255.255.255.252 | N/A |
|  | S0/1/1 (DCE) | 10.2.2.2 | 255.255.255.252 | N/A |
| RT3 | G0/0/1 | 172.16.3.1 | 255.255.255.0 | N/A |
|  | S0/1/1 | 10.2.2.1 | 255.255.255.252 | N/A |
| Server1 | NIC | 172.16.1.200 | 255.255.255.0 | 172.16.1.1 |
| Server2 | NIC | 172.16.1.201 | 255.255.255.0 | 172.16.1.1 |
| Admin-PC | NIC | 172.16.3.100 | 255.255.255.0 | 172.16.3.1 |

Objectives

Configure OSPF SHA256 authentication.

Configure NTP.

Configure routers to log messages to the syslog server.

Configure RT1 to support SSH connections.

Lock down a router using AutoSecure and verify the configuration.

Background / Scenario

In this activity, you will configure OSPF SHA authentication for secure routing updates.

The NTP Server is the master NTP server in this activity. You will configure authentication on the NTP server and the routers. You will configure the routers to allow the software clock to be synchronized by NTP to the time server. Also, you will configure the routers to periodically update the hardware clock with the time learned from NTP.

The Syslog Server will provide message logging in this activity. You will configure the routers to identify the remote host (Syslog server) that will receive logging messages.

You will need to configure timestamp service for logging on the routers. Displaying the correct time and date in Syslog messages is vital when using Syslog to monitor a network.

You will configure RT1 to be managed securely using SSH instead of Telnet. The servers have been pre-configured for NTP and Syslog services respectively.

You will also use automated security features on router R3.

The routers have been pre-configured with the following passwords:

Enable password: ciscoenpa55

Password for vty lines: ciscovtypa55

Configure OSPF SHA Authentication

Test connectivity. All devices should be able to ping all other IP addresses.

Create a SHA authentication key chain on RT1 for use with OSPF

Create a key chain called OSPFSHA on RT1 using the key chain command in order to enter key chain configuration mode.

Assign the key chain number 10 and set the password as sha256OSPF using the key and key-string commands respectively.

Specify SHA256 authentication with the cryptographic-algorithm command.

Assign the authentication key to the OSPF interfaces on RT1

Assign the previously created authentication key to the serial interface on RT1 using the ip ospf authentication key-chain command.

QUESTION 1 Wait for approx. 30 seconds and then attempt to ping from the Admin-PC to either of the servers. Were the pings successful and, if not, why?

Before proceeding, go to the “Lab 4 - Configuring OSPF Authentication, NTP, Syslog and SSH - QUESTIONS” quiz on the Brightspace page and enter your answer for question 1. Leave the quiz open while you complete the rest of the lab sheet.

Configure SHA authentication for OSPF on RT2 and RT3

Repeat steps 2 and 3 on RT2 and RT3 for all the active serial interfaces.

Verify configurations.

Verify the SHA256 authentication configurations using the commands show ip ospf interface.

Verify end-to-end connectivity.

QUESTION 2 Can Admin-PC ping the servers now?

- Before proceeding, return to the quiz on the Brightspace page and enter your answer for question 2. Leave the quiz open while you complete the rest of the lab sheet.

Configure NTP

Enable NTP authentication on Server2.

On Server2, click NTP under the Services tab to verify NTP service is enabled.

To configure NTP authentication, click Enable under Authentication. Use key 5 and password md5pa$$NTP for authentication.

Configure RT1, RT2, and RT3 as NTP clients.

Configure the three routers to obtain their time information from the NTP server.

Verify client configuration using the command show ntp status. (Note: You may have to wait several minutes to verify. You can click the “Fast Forward Time” button in the bottom corner to speed it up.)

QUESTION 3 Is the clock showing as synchronised?

QUESTION 4 What stratum number is the router currently showing as?

QUESTION 5 Based on this stratum number, what is the stratum number of the NTP server that the router is obtaining its time from?

- Before proceeding, return to the quiz on the Brightspace page and enter your answers for questions 3-5. Leave the quiz open while you complete the rest of the lab sheet.

Configure routers to update hardware clock.

Configure RT1, RT2, and RT3 to periodically update the hardware clock with the time learned from NTP.

Exit global configuration and verify that the hardware clock was updated using the command show clock.

Configure NTP authentication on the routers.

Configure NTP authentication on RT1, RT2, and RT3 using key 5 and password md5pa$$NTP.

Configure Routers to Log Messages to the Syslog Server

Configure routers to timestamp log messages.

Configure timestamp service for logging on the routers using the service timestamps command. Hint: Try using the context sensitive help (“?”) to figure out how to complete this command as necessary.

Configure the routers to identify the remote host (Syslog Server) that will receive logging messages.

Configure the routers to send log messages to the Syslog server and exit back to enable mode. The router console will display a message that logging has started.

Verify logging configuration.

Use the command show logging to verify logging has been enabled.

Examine logs of the Syslog Server.

From the Services tab of the Syslog Server’s dialogue box, select the Syslog services button. Observe the logging messages received from the routers.

Disable the LAN interface on either RT1 or RT3 and verify that a log message is sent to the Syslog server (make sure to re-enable the interface again).

QUESTION 6 What is the severity level of the log message(s) displayed when the interface is manually disabled?

QUESTION 7 What is the MNEMONIC on the log message?

Before proceeding, return to the quiz on the Brightspace page and enter your answers for questions 6 & 7. Leave the quiz open while you complete the rest of the lab sheet.

Configure RT1 to Support SSH Connections

Configure a minimum password length.

Configure a minimum password length of 12 character on RT1.

Configure a maximum number of failed login attempts before new attempts are disabled.

Configure RT1 to disable login attempts for a minute and a half when 5 failed login attempts are inputted within 30 seconds.

Configure log messages to be sent for unsuccessful login attempts.

Configure RT1 to send log messages for unsuccessful login attempts.

Configure a domain name.

Configure a domain name of netsec.com on RT1.

Configure users for login to the SSH server on RT1.

Create a user ID of AdminSSH and a secret password of ciscosshpa55.

Configure the incoming vty lines on RT1.

Use the local user accounts for mandatory login and validation. Accept only SSH connections.

Erase existing key pairs on RT1.

Any existing RSA key pairs should be erased on the router.

Note: If no keys exist, you might receive this message: % No Signature RSA Keys found in configuration.

Generate the RSA encryption key pair for RT1.

The router uses the RSA key pair for authentication and encryption of transmitted SSH data. Configure the RSA keys with a modulus of 2048.

Verify the SSH configuration.

Use the show ip ssh command to see the current settings.

QUESTION 8 What is the current SSH version?

QUESTION 9 What is the currently timeout (in seconds)?

QUESTION 10 What is the current number of authentication retries?

Before proceeding, return to the quiz on the Brightspace page and enter your answers for questions 8-10. Leave the quiz open while you complete the rest of the lab sheet.

Configure SSH timeouts and authentication parameters.

The default SSH timeouts and authentication parameters can be altered to be more restrictive. Set the timeout to 100 seconds, the number of authentication retries to 1, and the version to 2.

Issue the show ip ssh command again to confirm that the values have been changed.

Attempt to connect to RT1 via Telnet from Admin-PC.

Open the Desktop of Admin-PC. Select the Command Prompt icon. From Admin-PC, enter the command to connect to RT1 via Telnet.

PC> telnet 10.1.1.1

Connect to RT1 using SSH on Admin-PC.

Open the Desktop of Admin-PC. Select the Command Prompt icon. From Admin-PC, enter the command to connect to RT1 via SSH. When prompted for the password, enter the password configured for the administrator ciscosshpa55.

PC> ssh –l AdminSSH 10.1.1.1

QUESTION 11 Were the Telnet or SSH successful?

Verify failed login attempts are being monitored.

Close the SSH connection to RT1 on Admin-PC (if still open) and attempt to make a new connection but this time deliberately misspell the password.

Open the Services tab of the Syslog Server’s dialogue box, select the Syslog services button and verify if a log message has been received from RT1 relating to the failed login attempt.

Use the command show login failures on RT1 to display information about failed login attempts.

QUESTION 12 What is the source IP address showing on the output on RT1 for the most recent failed login attempt?

Before proceeding, return to the quiz on the Brightspace page and enter your answers for questions 11 & 12. Leave the quiz open while you complete the rest of the lab sheet.

Configure Automated Security Features

In this part, you will use AutoSecure to secure RT3. By using a single command in CLI mode, the AutoSecure feature allows you to disable common IP services that can be exploited for network attacks. It can also enable IP services and features that can aid in the defense of a network when under attack. AutoSecure simplifies the security configuration of a router and hardens the router configuration.

### Use the AutoSecure Cisco IOS feature on RT3.

      - Enter privileged EXEC mode using the enable command.

      - Issue the auto secure command on RT3 to lock down the router. The connection to RT2 represents a WAN connection, so assume that RT3 S0/1/1 is connected to the internet when prompted by the AutoSecure questions. Respond to the AutoSecure questions as shown in the following output. The responses are bolded.

Open configuration window

RT3# auto secure

--- AutoSecure Configuration ---

*** AutoSecure configuration enhances the security of

the router, but it will not make it absolutely resistant

to all security attacks ***

AutoSecure will modify the configuration of your device.

All configuration changes will be shown. For a detailed

explanation of how the configuration changes enhance security

and any possible side effects, please refer to Cisco.com for

Autosecure documentation.

At any prompt you may enter '?' for help.

Use ctrl-c to abort this session at any prompt.

Gathering information about the router for AutoSecure

Is this router connected to internet? [no]: yes

Enter the number of interfaces facing internet [1]: [Enter]

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0/0   unassigned      YES NVRAM  administratively down down

GigabitEthernet0/0/1   172.16.3.1      YES manual up                    up

Serial0/1/0            unassigned      YES NVRAM  administratively down down

Serial0/1/1            10.2.2.1        YES manual up                    up

Vlan1                  unassigned      YES unset  administratively down down

Enter the interface name that is facing internet: Serial0/1/1

Securing Management plane services...

Disabling service finger

Disabling service pad

Disabling udp & tcp small servers

Enabling service password encryption

Enabling service tcp-keepalives-in

Enabling service tcp-keepalives-out

Disabling the cdp protocol

Disabling the bootp server

Disabling the http server

Disabling the finger service

Disabling source routing

Disabling gratuitous arp

Here is a sample Security Banner to be shown

at every access to device. Modify it to suit your

enterprise requirements.

Authorized Access only

This system is the property of So-&-So-Enterprise.

UNAUTHORIZED ACCESS TO THIS DEVICE IS PROHIBITED.

You must have explicit permission to access this

device. All activities performed on this device

are logged. Any violations of access policy will result

in disciplinary action.

Enter the security banner {Put the banner between

k and k, where k is any character}:

# This service is restricted to authorised users only. All activities on this system are logged. #

Enable secret is either not configured or

is the same as the enable password

Enter the new enable secret: ciscoenpa55

Confirm the enable secret: ciscoenpa55

Enter the new enable password: 55ciscoenpa

Confirm the enable password: 55ciscoenpa

Configuration of local user database

Enter the username: AdminSSH

Enter the password: ciscosshpa55

Confirm the password: ciscosshpa55

Configuring AAA local authentication

Configuring console, Aux and vty lines for

local authentication, exec-timeout, transport

Securing device against Login Attacks

Configure the following parameters

Blocking Period when Login Attack detected: 90

Maximum Login failures with the device: 5

Maximum time period for crossing the failed login attempts: 30

Configure SSH server? [yes]: [Enter]

Enter the host name: RT3

Enter the domain-name: netsec.com

Disabling mop on Ethernet interfaces

Securing Forwarding plane services...

Enabling CEF (This might impact the memory requirements for your platform)

Enabling unicast rpf on all interfaces connected

to internet

Configure CBAC Firewall feature? [yes/no]: no

Tcp intercept feature is used prevent tcp syn attack

on the servers in the network. Create autosec_tcp_intercept_list

to form the list of servers to which the tcp traffic is to

be observed

Enable tcp intercept feature? [yes/no]: no

This is the configuration generated:

!

service password-encryption

no cdp run

access-list 100 permit udp any any eq bootpc

banner motd # This service is restricted to authorised users only. All activities on this system are logged. #

enable secret 5 $1$mERr$TfFTxE.mmb5O5BVC56ndL0

enable password 7 0874194D000A061817051C05

username AdminSSH password 7 0822455D0A1616041A1B0D517F

aaa new-model

aaa authentication login local_auth local

line con 0

login authentication local_auth

exec-timeout 5 0

transport output telnet

line vty 0 4

login authentication local_auth

transport input telnet

service timestamps debug datetime msec

service timestamps log datetime msec

logging trap debugging

logging console

logging buffered

line vty 0 4

transport input ssh

transport input telnet

hostname RT3

ip domain-name netsec.com

ip access-list extended 100

permit udp any any eq bootpc

Apply this configuration to running-config? [yes]: [Enter]

Applying the config generated to running-config

The name for the keys will be: test.test

% The key modulus size is 1024 bits

% Generating 1024 bit RSA keys, keys will be non-exportable...

*Mar  1 22:56:41.001: %SYS-3-CPUHOG: Task is running for (2007)msecs, more than

(2000)msecs (0/0),process = crypto sw pk proc.

-Traceback= 0x824198E0 0x82419FC4 0x8283C238 0x82866AD8 0x828667A8 0x82865D34 0x

828660F4 0x82866510 0x802335D4 0x80236D80 [OK]

RT3#

Note: On a productions system, the questions asked would vary depending on the features on the IOS image and device.

### Correct the OSPF authentication.

      - The AutoSecure configuration will have caused the OSPF authentication password to have been altered. In order to fix this, please re-do Part 1 Step 2.

If you have correctly configured all parts of the lab your activity score should now be showing as 100%. If so, click on “check results” in the activity window. Return to the Brightspace quiz one last time and enter the code into the appropriate question box (Q13) of the quiz.

You have completed the lab – please submit the Brightspace quiz.