# Lab 5 - Configure LAN Security

# Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A | Core G0/1 |
| S1 | VLAN 80 | 192.168.80.2 | 255.255.255.0 | 192.168.80.1 | N/A |
| S2 | VLAN 80 | 192.168.80.3 | 255.255.255.0 | 192.168.80.1 | N/A |
| Core | VLAN 80 | 192.168.80.4 | 255.255.255.0 | 192.168.80.1 | N/A |
| PC-A | NIC | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 | S1 F0/6 |
| PC-B | NIC | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 | S2 F0/18 |
| Admin | NIC | 192.168.80.100 | 255.255.255.0 | 192.168.80.1 | S2 F0/3 |

# VLAN Table

| VLAN | Name | Interface Assigned |
| --- | --- | --- |
| 20 | Sales | S1: F0/6 S2: F0/18 |
| 80 | Admin | Core: VLAN 80 S1: VLAN 80 S2: F0/3, VLAN 80 |
| 199 | Native | N/A |
| 999 | Nowhere | Core: F0/3-24, G0/2 S1: F0/2-4, F0/7-24, G0/1-2 S2: F0/4-17, F0/19-24, G0/1-2 |

# Objectives

Part 1: Configure Basic Switch Settings

Build the topology.

Configure the hostname, IP address, and configure SSH.

Part 2: Configure Secure Trunks Ports and Create VLANs

Configure trunk port mode.

Change the native VLAN for trunk ports.

Verify trunk configuration.

Disable trunking.

Part 3: Create VLANs and Assign Switch Ports

Part 4: Configure Port Security and Disable Unused Ports

Configure and verify port security.

Disable unused ports.

# Background / Scenario

The Layer 2 infrastructure consists mainly of interconnected Ethernet switches. Most end-user devices, such as computers, printers, IP phones, and other hosts, connect to the network via Layer 2 access switches. As a result, switches can present a network security risk. Similar to routers, switches are subject to attack from malicious internal users. The switch Cisco IOS software provides many security features that are specific to switch functions and protocols.

In this lab, you will configure various switch protection measures, including secure VLANs and access port security.

Note: Before you begin, ensure that the routers and the switches have been erased and have no startup configurations.

# Required Resources

1 Router (Cisco 4221 with Cisco XE Release 16.9.6 universal image or comparable with a Security Technology Package license)

3 Switches (Cisco 2960+ with Cisco IOS Release 15.2(7) lanbasek9 image or comparable)

2 PCs (Windows OS with a terminal emulation program, such as PuTTY or Tera Term installed)

Console cables to configure Cisco networking devices

Ethernet cables as shown in the topology

# Instructions

## Configure Basic Switch Settings

In Part 1, you will set up the network topology and configure basic settings, such as the hostnames, IP addresses, and device access passwords.

### Cable the network as shown in the topology.

Attach the devices, as shown in the topology diagram, and cable as necessary.

### Configure basic settings for the router and each switch.

Perform all tasks on R1, Core, S1, and S2. The procedure for S1 is shown here as an example.

      - Configure hostnames, as shown in the topology.

Switch(config)# hostname S1

      - Configure interface IP addresses, as shown in the IP Addressing Table. The following configuration displays the VLAN 80 management interface on S1:

S1(config)# interface vlan 80

S1(config-if)# ip address 192.168.80.2 255.255.255.0

S1(config-if)# exit

S1(config)# ip default-gateway 192.168.80.1

      - On R1 issue the command username SSHadmin algorithm-type ? to view the different password hashing schemes that are available to secure passwords stored in configuration files.

- R1(config)# username SSHadmin algorithm-type ?

QUESTION 1 What algorithm types are available for hashing configured passwords?

Before proceeding, go to the “Lab 5 - Configure LAN Security (HW) - QUESTIONS” quiz on the Brightspace page and enter your answer for question 1. Leave the quiz open while you complete the rest of the lab sheet.

      - Create a username SSHadmin and password ciscosshpa55 with Scrypt hashing on R1.

      - Configure R1 for SSH connectivity. Use the domain name lab5.com and configure the RSA keys with a modulus of 2048.

### Configure PC host IP settings.

Configure a static IP address, subnet mask, and default gateway for PC-A, PC-B and the Admin PC, as shown in the Addressing Table.

### Verify basic network connectivity.

Ping from PC-A and PC-B to the R1 G0/1 interface at IP address 192.168.1.1.

If the pings are unsuccessful, troubleshoot the basic device configurations before continuing.

Ping from PC-A to PC-B.

If the pings are unsuccessful, troubleshoot the basic device configurations before continuing.

SSH from PC-A to R1.

PC> ssh –l SSHadmin 192.168.1.1

Password: <ciscosshpa55>

R1>

If the SSH connection is unsuccessful, troubleshoot the basic device configurations before continuing.

## Configure Secure Trunk Ports

In this part, you will configure trunk ports, change the native VLAN for trunk ports, and verify trunk configuration.

Securing trunk ports can help stop VLAN hopping attacks. The best way to prevent a basic VLAN hopping attack is to explicitly disable trunking on all ports except the ports that specifically require trunking. On the required trunking ports, disable DTP (auto trunking) negotiations and manually enable trunking. If no trunking is required on an interface, configure the port as an access port. This disables trunking on the interface.

Note: Tasks should be performed on Core, S1 or S2, as indicated.

### Configure trunk ports on Core, S1 and S2.

Configure the links between the 3 switches for trunking by setting the necessary ports to trunk mode. Configuration for Core is shown below as an example.

Core(config)# interface range F0/1-2

Core(config-if-range)# switchport mode trunk

Verify the trunk links with the show interfaces trunk command.

S1# show interfaces trunk

Port     Mode         Encapsulation  Status        Native vlan

Fa0/1    on           802.1q         trunking      1

Fa0/5    on           802.1q         trunking      1

Port     Vlans allowed on trunk

Fa0/1    1-4094

Fa0/5    1-4094

Port     Vlans allowed and active in management domain

Fa0/1    1

Fa0/5    1

Port     Vlans in spanning tree forwarding state and not pruned

Fa0/1    1

Fa0/5    1

QUESTION 2 What is the current native VLAN on the switches?

Before proceeding, return to the quiz and enter your answer for question 2. Leave the quiz open while you complete the rest of the lab sheet.

### Change the native VLAN for the trunk ports on Core, S1 and S2.

Changing the native VLAN for trunk ports to an unused VLAN helps prevent VLAN double-tagging attacks. Set the native VLAN on the trunk interfaces on Core to VLAN 199.

Core(config)# interface range F0/1-2

Core(config-if-range)# switchport trunk native vlan 199

The following message should display after a brief period of time:

- %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/2 (199), with S2 FastEthernet0/2 (1).

- %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (199), with S1 FastEthernet0/5 (1).

Set the native VLAN on the S1 & S2 trunk interfaces to VLAN 199.

### Prevent the use of DTP on Core, S1 and S2.

Setting the trunk ports to nonegotiate also helps to mitigate VLAN hopping by turning off the generation of DTP frames. Disable DTP on the trunk links between the 3 switches. Configuration for Core is shown below as an example.

Core(config)# interface range F0/1-2

Core(config-if-range)# switchport nonegotiate

### Verify the trunking configurations.

S1# show interfaces F0/1 trunk

Port     Mode         Encapsulation  Status        Native vlan

Fa0/1    on           802.1q         trunking      199

Port     Vlans allowed on trunk

Fa0/1    1-4094

Port     Vlans allowed and active in management domain

Fa0/1    1

Port     Vlans in spanning tree forwarding state and not pruned

Fa0/1    1

S1# show interfaces F0/1 switchport

Name: Fa0/1

Switchport: Enabled

Administrative Mode: trunk

Operational Mode: trunk

Administrative Trunking Encapsulation: dot1q

Operational Trunking Encapsulation: dot1q

Negotiation of Trunking: Off

Access Mode VLAN: 1 (default)

Trunking Native Mode VLAN: 199 (Inactive)

Administrative Native VLAN tagging: enabled

Voice VLAN: none

Administrative private-vlan host-association: none

Administrative private-vlan mapping: none

Administrative private-vlan trunk native VLAN: none

Administrative private-vlan trunk Native VLAN tagging: enabled

Administrative private-vlan trunk encapsulation: dot1q

Administrative private-vlan trunk normal VLANs: none

Administrative private-vlan trunk private VLANs: none

Operational private-vlan: none

Trunking VLANs Enabled: ALL

Pruning VLANs Enabled: 2-1001

Capture Mode Disabled

Capture VLANs Allowed: ALL

Protected: false

Unknown unicast blocked: disabled

Unknown multicast blocked: disabled

Appliance trust: none

### Verify the configuration with the show run command.

Use the show run command to display the running configuration, beginning with the first line that has the text string “0/1” in it.

S1# show run | begin 0/1

interface FastEthernet0/1

switchport trunk native vlan 199

switchport mode trunk

switchport nonegotiate

<output omitted>

## Create VLANs and Enable Admin VLAN

The network administrator wants to access all switch and routing devices using an Admin PC. For security purposes, the administrator wants to ensure that all managed devices are on a separate VLAN.

In Part 3, you will create VLANs as specified in the table above on all 3 switches. You will then assign the VLANs to the appropriate interface and verify your configuration settings. Complete the following tasks on each switch.

    - Create VLANs on each switch.

      - On all 3 switches, create the VLANs outlined in the VLAN table at the top of the lab sheet.

    - Assign VLANs to the correct switch interfaces.

      - Assign used ports on S1 and S2 to the appropriate VLAN (specified in the VLAN table above) and configure them for static access mode only.

- Note: Configuring the ports as access mode means that they are explicitly non-trunking so are not susceptible to VLAN hoping attacks. Placing used ports in dedicated VLANs (rather than leaving them in the default VLAN) also helps mitigate VLAN double-tagging attacks.

      - Assign all unused ports on the switches to the Nowhere VLAN, configure them for static access mode, and administratively deactivate them. The configuration is shown below for S2 as an example.

S2(config)# interface range F0/4 - 17 , F0/19 - 24 , G0/1 - 2

S2(config-if-range)# switchport mode access

S2(config-if-range)# switchport access vlan 999

S2(config-if-range)# shutdown

    - Verify connectivity of the management PC to all switches.

The Admin PC should be able to ping S1, S2, and Core.

QUESTION 3 Which devices, if any, was Admin able to successfully ping?

Before proceeding, return to the quiz and enter your answer for question 3. Leave the quiz open while you complete the rest of the lab sheet.

    - Enable new subinterfaces on router R1.

      - Create subinterface G0/1.80 and set encapsulation to dot1q 80 to account for VLAN 80.

R1(config)# int G0/1.80

R1(config-subif)#

%LINK-5-CHANGED: Interface GigabitEthernet0/1.80, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1.80, changed state to up

R1(config-subif)# encapsulation dot1q 80

      - Assign an IP address of 192.168.80.1 /24 to the subinterface.

- R1(config-subif)# ip address 192.168.80.1 255.255.255.0

      - Create subinterface G0/1.1 and set encapsulation to dot1q 1 to account for VLAN 1.

- R1(config)# int G0/1.1
R1(config-subif)# encapsulation dot1q 1

      - Assign the IP address 192.168.1.1. (You will have to remove it from the G0/1 interface first using the no ip address command).

- R1(config)# int G0/1
R1(config-if)# no ip address
R1(config-if)# int G0/1.1
R1(config-subif)# ip address 192.168.1.1 255.255.255.0

    - Create a trunk link from Core to R1.

- Change G0/1 on Core to be a trunk port. Disable DTP negotiation and change the native VLAN to 199.

- Core(config)# int G0/1
Core(config-if)# switchport mode trunk 
Core(config-if)# switchport trunk native vlan 199
Core(config-if)# switchport nonegotiate

    - Verify SSH.

      - Verify the Admin PC can access the router. Use SSH to access R1 with username SSHadmin and password ciscosshpa55.

PC> ssh -l SSHadmin 192.168.80.1

QUESTION 4 Can Admin SSH to R1?

Before proceeding, return to the quiz and enter your answer for question 4. Leave the quiz open while you complete the rest of the lab sheet.

## Configure Port Security and Disable Unused Ports

Switches can be subject to a CAM table, also known as a MAC address table, overflow, MAC spoofing attacks, and unauthorized connections to switch ports. In this task, you will configure port security to limit the number of MAC addresses that can be learned on a switch port and disable the port if that number is exceeded.

    - Configure basic port security on all ports connected to host devices.

This procedure should be performed on the access ports on S1 and S2.

- Enable port security on the access ports on S1 and S2.

Note: A switch port must be configured as an access port to enable port security.

- Set the maximum number of learned MAC addresses to 2.

- Configure the ports so that any learned MAC addresses will be added to the running configuration (i.e. sticky).

- Set the violation mode so that the ports are not disabled when a violation occurs but the offending packet is dropped and a log message is generated.

Hint: See page 20 of lecture 7 for details of the different violation modes.

    - Verify port security.

      - On S1, issue the command show port-security interface F0/6 to verify that port security has been configured.

S1# show port-security interface F0/6

Port Security              : Enabled

Port Status                : Secure-up

Violation Mode             : <hidden>

Aging Time                 : 0 mins

Aging Type                 : Absolute

SecureStatic Address Aging : Disabled

Maximum MAC Addresses      : 2

Total MAC Addresses        : 0

Configured MAC Addresses   : 0

Sticky MAC Addresses       : 0

Last Source Address:Vlan   : 0000.0000.0000:0

Security Violation Count   : 0

      - Ping from PC-A to PC-B and issue the command show port-security interface F0/6 again to verify that the switch has learned the MAC address for PC-A.

QUESTION 5 What is the “Total MAC Addresses” value in the output after doing the ping?

Return to the quiz and enter your answer for question 5.

You have completed the lab – get your lab supervisor to correct your work and submit the Brightspace quiz.